Description of main.c

Updated by: Faris

Updated on: 27 Sep 2025

File uploaded as: “main(2).c”

Basic Information

The code is made of 3 things:

- Generated code (can be ignored)

- Global variables (user defined)

- Functions (user defined)

- Tasks (user defined)

Difference btw task and function

- Function: Works like a normal function that do some steps when you call it

- Task: Created from RealRTOS which allows many different tasks to run simultaneously. Used to get sensor readings

Note: Some of the global variables, functions and tasks are not in used anymore but are kept in case need to refer to them again.

Code Structure

#---------------- Generated code ----------------
Line 1 – 184 (can be ignored)
#------------------------------------------------------

#-------- User defined global variables --------
Line 185 – 334
#------------------------------------------------------

#---------------- Generated code ----------------
Line 335 – 1145 (can be ignored)
#------------------------------------------------------

#----------- User defined functions ------------
Line 1146 - 1771
#------------------------------------------------------

#-------------- User defined tasks ---------------
Line 1172 - 2361
#------------------------------------------------------

#---------------- Generated code ----------------
Line 2361 - end
#------------------------------------------------------

Functions

- “delay_us()”

- Give delay in microseconds which is not possible with basic delay functions

- “HAL_TIM_IC_CaptureCallback()”

- For ultrasonic sensor, used to calculate time difference btw sending and receiving ultrasonic wave

- Motor_direction_both()

- If parameter = 1, set motor to move forward, else reverse

- “display_if_changed()”

- Displays message on OLED if the string has changed

- HAL_UART_RxCpltCallback()

- Reads incoming UART messages, displays on OLED when receives ‘\n’

- “convert_instruction()”

- Reads instruction and convert them to correct type for processing

- I2C_Bus_Recovery()

- The I2C connection needs to be refreshed occasionally, so this function refreshes it

- “icm20948_init()”

- Initializes the IMU and displays the status on OLED

- PID_Compute()

- Calculates the expected pwm value for motor given a target speed using a PID controller

- “forward()”

- Moves forward at a target speed for a target distance

- Parameter “delay” is the interval between each PID_Compute() call

- Parameter “flag” is for the stopping

- “forward_until()”

- Same as forward() but moves forward until target_distance from an object

- “reverse()”

- Same as forward, but reverse

- “reverse_until()”

- Same as forward_until() but in reverse

- “forward_turn()”

- Parameter “dir” is the pwm value for the servo, used to tell turn left or right

- Parameter “target_angle” used to show how much to turn

- Parameter “target_rpm” and “delay” are kept constant and used to call the forward() function

- Outside rear wheel spins more than  inside rear wheel

- “reverse_wheel()”

- Same as forward_turn() but in reverse, rear wheel spins backwards

- “buzzerOn()”

- On the buzzer for certain timing set by parameter

- “rotate_wheel()”

- Rotate wheel to a set position by mentioning pwm value in parameter

- Easier to use than remembering the exact function call

- “convert_from_angle()”

- From a given angle, converts to a pwm value which is used to set the servo (front wheels)

- Negative angle means left, positive means right

Tasks

- StartDefaultTask()

- Blink the LED to show everything is running and the code is not stuck

- “show()”

- Used to display and refresh the OLED

- “motors”

- When receive instruction, will execute it

- “encoder”

- Calculate the rpm of right motor (motor A)

- “encoder_B”

- Calculate the rpm of left motor (motor B)

- “ultra_sensor()”

- Calculate the distance btw object and ultrasensor

- “sensor_reading()”

- Starts with 1 second delay to calculate average noise at rest

- Calculate angle using angular velocity from gyroscope reading

- “ir_dist()”

- Calculate the distance btw ir sensor and obstacle on the left and right

- “transmit()”

- Currently not in use

- Used to transmit on UART

- “encoder_diff”

- Currently not in use

- Calculate the encoder difference between motor A and B (which wheel spin more)